SEWAGE SLUDGE MANAGEMENT

- HOUSEHOLDERS

  - I need to empty my septic tank, what should I do?

Septic tanks should be pumped when sludge and scum accumulation exceed 30% of the tank volume or are encroaching on the inlet and outlet baffle entrances. Periodic pumping of septic tanks is recommended to ensure proper system performance and reduce the risk of hydraulic failure. Septic tanks should be de-sludged at a minimum of once every year, in cases where the septic tank is at, or near, its design load capacity, de-sludging should be more often if the rate of sludge building-up required more frequent removal. Accumulated sludge and scum material found in the tank should be removed by an appropriately permitted contractor in accordance with the Waste Management (Collection Permit) Regulations 2001.  (see table below for some exceptions to the requirement to hold such a permit).

The local authority have a list of permitted contractors in the area. The permitted contractor will arrange for the disposal of the sludge in accordance with the national legislations via either disposal to agriculture or disposal to a managed wastewater treatment municipal facility. Householders should obtain a certificate from the permitted contractor each time their tank is de-sludged.

- CONTRACTORS/FARMERS/SLUDGE REMOVERS

  - What permits/authorisations do I require to empty septic tanks/waste water treatment systems?

    - Firstly, you must hold a Waste Collection Permit issued by the Local Authority, or from 1st February 2012 by the National Waste Collection Permit Office (nwcpo@offalycoco.ie), under the Waste Management (Collection Permit) Regulations 2001 to transport off site any waste, including sewage sludge from septic tanks or treatment systems.  See table below for a list of scenarios where such permits are/are not required.

    - Disposal/Recycling of the sludge material will then either be by:

- landspreading it in accordance with the legislation detailed in item 1) below, or

- bringing it to an Environmental Protection Agency (EPA) Licensed Wastewater Treatment Plant which accepts this material.  Such facilities located in Kildare are listed in item 2) below.

1). Sludge from a septic tank or a sewage treatment system that is intended to be directly landspread should be managed in accordance with the Waste Management (Use of Sewage Sludge in Agriculture) Regulations, 1998 and its amendment Waste Management (Use of Sewage Sludge in Agriculture) (Amendment) Regulations, 2001.  These regulations allow for the landspreading of untreated sewage sludge on grass land and on tillage land once it is worked into the land, provided that certain criteria are met and that it is carried out in accordance with the Nutrient Management Plan for the lands in question.  Such Nutrient Management Plans must be submitted to the Local Authority for approval prior to any landspreading taking place and must also comply with the European Communities (Good Agricultural Practice for Protection of Waters) Regulations 2010.  

2). Alternatively, sewage sludge can be brought to approved Waste Water Treatment Plants.  In Kildare there are 3 Wastewater Treatment Plants that will accept sewage sludge; Athy WWTP, Leixlip WWTP and Osberstown WWTP.  Contractors should contact the Treatment Plant beforehand to confirm acceptance and arrange delivery.

  - Can a farmer empty a septic tank for a neighbour, and spread it on his land?

No. The collection of wastes by any person or business for the purpose of profit, reward or otherwise in the course of business requires a waste collection permit under the Waste Management (Collection Permit) Regulations, 2001.  From the 1st February 2012, Offaly County Council are the Nominated Authority of the processing of all new Waste Collection Permit applications and review applications.  The National Waste Collection Permit Office can be contacted on 057-9357428 or nwcpo@offalycoco.ie.  Farmers and farm contractors fall within the scope of this definition, and householders should always check in advance that the contractors they use have the required permits.

The following table is a summary of the various scenarios that may arise, and whether a collection permit is required in each scenario.

Please note that in addition to whether a Collection Permit is required, or not, the relevant legislation regarding landspreading (Waste Management (Use of Sewage Sludge in Agriculture) Regulations, 1998, as amended in 2001 and European Communities (Good Agricultural Practice for Protection of Waters) 2010) must also be complied with.

| Scenario | Collection Permit Requirement? | Relevant Section in Collection Permit Regulations |
| --- | --- | --- |
| Farmer collecting waste on his own land, and spreading it on his own land, including outlying lands in the same tenure. | No permit required. | Gathered, sorted or mixed on the premises where it arose. |
| Contractor spreading waste for a farmer, on the farm on which the waste arose, including outlying lands in the same tenure. | No permit required. | Gathered, sorted or mixed on the premises where it arose. |
| Contractor collecting waste from a farm and transporting it for disposal or recovery on another premises. | Permit required. | Section 34: Collection for the purpose of reward. |
| Farmer collecting waste not owned by him or her, and transporting it for recovery or disposal on another premises. | Permit required. | Section 34: Collection for the purpose of reward. |
| Contractor collecting septic tank sludge for recovery or disposal off site. | Permit required. | Section 34: Collection for the purpose of reward. |
| Farmer transporting wastes or excavation spoil for recovery off site. | Permit required. | Section 34: Collection for the purpose of reward. |
| Contractor collecting wastes from an Integrated Pollution Control (IPC)-licensed facility, and spreading on his or her own, or licensee-owned, or independently owned, spread lands. | Permit required. | Section 34: Collection for the purpose of reward. |
| Farmer collecting wastes from an IPC facility and spreading it on his or her own land. | Permit required. | Section 34: Collection for the purpose of reward. |

  - What are the main requirements in the legislation governing the management of septic tank/treatment systems sewage sludge?

- Water Services (Amendment) Act 2012 No. 2 of 2012

This Act amends specific Sections  the Water Services Act 2007 and also has sections on the following:

  - the requirement to register domestic waste water treatment systems.

  - the duties of owners of premises connected to domestic waste water treatment systems,

  - the inspection process,

  - the appointment of inspectors,

  - the register of inspectors,

  - the powers of inspectors,

  - the inspection process,

  - the supervision of the Agency of Water Services Authorities,

  - the direction to inspectors,

  - the National Inspection Plan,

  - Regulations, and

  - Offences and penalties

- Water Services Acts 2007 and 2012 Domestic Waste Water Treatment Systems (Registration) Regulations 2012, SI No. 220 of 2012

This legislation details the requirement to Register domestic waste water 		treatment systems, including

  - the prescribed date by which this must be done (1st February 2013),

  - the registration fee (€5 before 28th September 2012, and €50 thereafter),

  - the means of application (to the relevant water services authority in writing, or electronically via www.protectourwater.ie),

  - information required in the application (name of owner, correspondence address of owner, address of treatment system),

  - method of payment (in writing- by cheque, bank draft, postal order or credit/debit card, or also by cash when fee is being paid in person to the water services authority, or finally by credit/debit card when fee is being paid via the internet)

- Water Services (Amendment) Act 2012 (Commencement) Order 2012, SI No. 219 of 2012

This legislation gives effect to the Act, and appoints 26th June 2012, as the 	commencement date

- Water Services Acts 2007 and 2012 (Domestic Waste Water Treatment Systems) Regulations 2012, SI No. 223 of 2012

This legislation details information on the Operation and Maintenance of domestic waste water treatment systems and also on de-sludging.  Of particular note are the following sections:

- Section 2(1): A domestic waste water treatment system shall be operated and maintained by its owner so that domestic waste water or sewage effluent shall not emit, discharge, seept, leak or otherwise escape from the system, or part thereof;

- Section 2(2):  Roof water or surface water run-off shall not enter a domestic waste water treatment system

- Section 2(3):  The owner of a domestic waste water treatment system shall be responsible for its maintenance and renewal and shall ensure that its parts and components are fit for purpose, operational where appropriate and kept in good order and repair so as to prevent a risk to human health or the environment.

- Section 3(1):  A domestic waste water treatment system shall be de-sludged at intervals appropriate to the tank capacity and the number of persons resident in the premises connected to it or as recommended by the systems manufacturer.

- Section 3(2):  De-sludging shall be carried out by a contractor authorised under the Waste Management (Collection Permit) Regulations 2007 (SI No. 820 of 2007), as amended

- Section 3(3):  An owner shall obtain evidence of de-sludging or a receipt from the authorised contractor each time their tank is de-sludged and such evidence or receipt shall be retained for a period of five years

- Section 4(4):  .....the owner of a domestic waste water treatment system may carry out de-sludging of that system and use its contents in agriculture, subject to compliance with all relevant national legislative requirements  (see below)

- S.I. No. 148 of 1998- Waste Management (Use of Sewage Sludge in Agriculture) Regulations, 1998, as amended (S.I. No. 267 of 2001)

- Septic Tank sludge is not treated sludge and therefore cannot be used in agriculture (Article 3 (2)) unless it can be injected or otherwise worked into the land (Article 3 (3)).

- There is one exception however, as it can be applied to grassland provided that the grassland is not grazed within 6 months following landspreading.  It cannot be directly applied to any other crops. (Article 3 (4)).

- Its use on grassland requires a Nutrient Management Plan, and also the requirements set out in Article 4.

- It does not require returns to the Local Authority for inclusion in the Sludge Register. (Article 9)

- Soil Analysis must be carried out in accordance with Articles 6 and 7(1), and specifically those set out in Parts I, II and III.

- Sludge Analysis must be carried out in accordance with Article 7(2), as detailed in Part IV.  Section 6 of Part IV should be noted as it requires that the sludge analysis be carried out within six months after the commencement of the use of such sludge in agriculture.

- Waste Management Act, 1996, as amended by Article 23 of 2007 Facility Permit Regulations:

- A Waste Licence is not required for the recovery of sludge when used in agriculture

- Waste Management Act 1996-2009 and S.I. 820 of 2007-Waste Management (Collection Permit) Regulations 2007:

- A Waste Collection Permit is required to transport septic tank sludge -Articles 32 to 36 of the Act, specifically Art 34

- Other relevant legislation, guidance and codes of practice relating to landspreading:

- European Communities (Good Agricultural Practice for Protection of Waters) Regulations 2010 (S.I. No. 610 of 2010)

- Code of Good Practice for the Use of Biosolids in Agriculture, Department of the Environment

- S.I. No. 821 of 2007: Waste Management (Facility Permit and Registration) Regulations 2007 and amendment S.I. No. 86 of 2008

- Waste Management (Registration of Sewage Sludge Facility) Regulations 2010, S.I. No 32/2010

- S.I. No. 18/1996: Local Government (Water Pollution) Act, 1977

- S.I. No 108/1978: Local Government (Water Pollution) Regulations, 1978

- S.I. No. 270/1992: Local Government (Water Pollution)(Amendment) Act, 1990

- S.I. No. 271/1992: Local Government (Water Pollution) Regulations, 1992

- S.I. No. 184/1996: Local Government (Water Pollution)(Amendment)

Regulations, 1996.

- S.I. No. 42/1999: Local Government (Water Pollution)(Amendment) Regulations, 1999.

Note:  Kildare County Council have developed a document entitled 'Protocol for the use of Sewage Sludge in Agriculture', and is available on the Council website (www.kildare.ie/countycouncil/Environment/Farmingandtheenvironment.ie).  It details the requirements for landspreading treated sewage sludge (Biosolids) arising in Waste Water Treatment Plants.